High Efficiency Synchronous Boost Converter

With Dual Independent 1.5A Current Sources

- General Description

The OCP81373 is a dual LED flash driver that provides a high level of adjustability within a small solution size. The OCP81373 utilizes a 2MHz or 4MHz fixed-frequency synchronous boost converter to provide power to the dual 1.5A constant current LED sources. The total LED current the OCP81373 boost can deliver is 2A (ILED1+ ILED2). The dual 128 level current sources provide the flexibility to adjust the current ratios between LED1 and LED2 with each driver capable of delivering a maximum of 1.5A. An adaptive regulation method ensures the current sources remain in regulation and maximizes efficiency.

Features of the OCP81373 are controlled via an I2C-compatible interface. These features include: hardware flash and hardware torch pins (STROBE and TORCH/TEMP), a TX interrupt, and an NTC thermistor monitor. The device offers independently programmable currents in each output leg to drive the LEDs in a Flash or Movie Mode (Torch) condition.

The 2MHz or 4MHz switching frequency options, overvoltage protection (OVP), and adjustable current limit allow for the use of tiny, low-profile inductors and (10µF) ceramic capacitors. The device operates over a –40°C to 85°C ambient temperature range.

- Features

- 2A Total Allowed LED Current During

Operation

- Dual Independent LED Current Source

Programmability

- Accurate and Programmable LED Current

Range from 2.92mA to 1.5A

- Optimized Flash LED Current During Low

Battery Conditions (IVFM)

- Grounded Cathode LED Operation for Improved

Thermal Management

- Small Solution Size:<16mm2

- Hardware Strobe Enable (STROBE)

- Synchronization Input for RF Power Amplifier

Pulse Events (TX)

- Hardware Torch Enable (TORCH/TEMP)

- Remote NTC Monitoring (TORCH/TEMP)

- 400-kHz I2C-Compatible Interface

–OCP81373(I2C Address=0x63)

- WLCSP-12B

- Application

- Smart Phone White LED Flash

- Digital Still Cameras

- Pin Configuration

Figure 1, Pin Assignments of OCP81373（Top View）

| Pin Name | Pin No. | I/O | Pin Function |
| --- | --- | --- | --- |
|  | WLCSP-12B |  |  |
| GND | A1 | P | Ground |
| IN | A2 | I | Input voltage connection. Connect IN to the input supply and bypass to GND with a 10µF or larger ceramic capacitor. |
| SDA | A3 | I/O | Serial data input/output in the I2C Mode on OCP81373. |
| SW | B1 | P | Drain Connection for Internal NMOS and Synchronous PMOS Switches. |
| STROBE | B2 | I/O | Active high hardware flash enable. Drive STROBE high to turn on Flash pulse. Internal pulldown resistor 300kΩ between STROBE and GND. |
| SCL | B3 | I/O | Serial clock input for OCP81373 |
| OUT | C1 | O | Step-up DC/DC Converter Output. Connect a 10µF ceramic capacitor between this terminal and GND. |
| HWEN | C2 | I | Active high enable pin. High = Standby, Low = Shutdown/Reset. Internal pulldown resistor of 300kΩ between HWEN and GND. |
| TORCH/TEMP | C3 | I | Torch terminal input or threshold detector for NTC temperature sensing and current scale back. |
| LED2 | D1 | O | High-side current source output for flash LED. |
| TX | D2 | I | Configurable dual polarity power amplifier synchronization input. Internal pulldown resistor of 300kΩ between TX and GND. |
| LED1 | D3 | O | High-side current source output for flash LED. |

- Typical Application Circuit

Figure 2, Typical Application Circuit of OCP81373

- Block Diagram

Figure 3, Block Diagram of OCP81373